# ELE432 Advanced Digital Design

Assignment 1: Traffic Light Controller FSM - Simulation Report

# 1. Overview

This document presents the simulation results for the 4-state Traffic Light Controller FSM implemented in SystemVerilog. The design includes a 5-clock-cycle hold delay on yellow light states (S1 and S3), controlled by an internal counter.

# 2. FSM State Description

## State Encoding

Light output encoding: 2'b10 = GREEN,  2'b01 = YELLOW,  2'b00 = RED

## States

S0 (00):  LA = GREEN,  LB = RED   - Stay while TAORB=1. Transition to S1 when TAORB=0.

S1 (01):  LA = YELLOW, LB = RED   - Hold 5 cycles (timer). Transition to S2 when timer=5.

S2 (10):  LA = RED,    LB = GREEN - Stay while TAORB=0. Transition to S3 when TAORB=1.

S3 (11):  LA = RED,    LB = YELLOW - Hold 5 cycles (timer). Transition to S0 when timer=5.

# 3. Simulation Setup

Tool: Questa Altera Starter FPGA Edition 2025.2

Files: Traffic.sv (design), tb_Traffic.sv (testbench)

Clock period: 10 ns

Commands used:

vlog Traffic.sv tb_Traffic.sv

vsim -voptargs="+acc" work.tb_Traffic

add wave *

run -all

# 4. Transcript Output (Benchmark)

The following output was produced by the testbench during simulation:

=== Traffic Light FSM Simulation ===

--- S0: LA=Green, LB=Red (TAORB=1, stay) ---

t=21000 | TAORB=1 | LA=GREEN  | LB=RED

t=31000 | TAORB=1 | LA=GREEN  | LB=RED

t=41000 | TAORB=1 | LA=GREEN  | LB=RED

--- Transition to S1: TAORB -> 0 ---

t=51000 | TAORB=0 | LA=YELLOW | LB=RED

t=61000 | TAORB=0 | LA=YELLOW | LB=RED

t=71000 | TAORB=0 | LA=YELLOW | LB=RED

t=81000 | TAORB=0 | LA=YELLOW | LB=RED

t=91000 | TAORB=0 | LA=YELLOW | LB=RED

t=101000 | TAORB=0 | LA=RED   | LB=GREEN

--- S2: LA=Red, LB=Green (TAORB=0, stay) ---

t=111000 | TAORB=0 | LA=RED   | LB=GREEN

t=121000 | TAORB=0 | LA=RED   | LB=GREEN

t=131000 | TAORB=0 | LA=RED   | LB=GREEN

--- Transition to S3: TAORB -> 1 ---

t=141000 | TAORB=1 | LA=RED   | LB=YELLOW

t=151000 | TAORB=1 | LA=RED   | LB=YELLOW

t=161000 | TAORB=1 | LA=RED   | LB=YELLOW

t=171000 | TAORB=1 | LA=RED   | LB=YELLOW

t=181000 | TAORB=1 | LA=RED   | LB=YELLOW

t=191000 | TAORB=1 | LA=GREEN  | LB=RED

--- Back to S0: LA=Green, LB=Red ---

t=201000 | TAORB=1 | LA=GREEN  | LB=RED

t=211000 | TAORB=1 | LA=GREEN  | LB=RED

t=221000 | TAORB=1 | LA=GREEN  | LB=RED

=== Simulation Complete ===

# 5. Waveform Output

The waveform below shows all signals over the full simulation. LA and LB transition correctly through all 4 states with 5-cycle yellow hold confirmed.

# 6. Verification Summary

Compile result: Errors: 0, Warnings: 0

S0 (LA=GREEN, LB=RED) held correctly while TAORB=1: PASS

S1 yellow hold for exactly 5 cycles before S2: PASS

S2 (LA=RED, LB=GREEN) held correctly while TAORB=0: PASS

S3 yellow hold for exactly 5 cycles before S0: PASS

Full cycle S0-S1-S2-S3-S0 completed successfully: PASS